A PROJECT REPORT

ON

“Autonomous AI in Navigation and Visualization”

A project report submitted in partial fulfillment of the requirements for the degree of Bachelor of Technology in Artificial Intelligence & Machine Learning

BACHELOR OF TECHNOLOGY IN ARTIFICIAL INTELLIGENCE & MACHINE LEARNING

Submitted By

Sanskar Jadhav

PRN 21070126076

UNDER THE GUIDANCE OF

Prof. Kalyani Kadam

Assistant Professor

Pune – 412115, Maharashtra State, India
https://www.sitpune.edu.in/

DEPARTMENT OF ARTIFICIAL INTELLIGENCE & MACHINE LEARNING

AY 2024-25

Pune – 412115, Maharashtra State, India
https://www.sitpune.edu.in/

DEPARTMENT OF ARTIFICIAL INTELLIGENCE & MACHINE LEARNING

CERTIFICATE

This is to certify that the Project work entitled “Autonomous AI in Navigation and Visualization” is carried out by Sanskar Jadhav, in partial fulfillment for the award of the degree of Bachelor of Technology in Artificial Intelligence & Machine Learning, Symbiosis International (Deemed University), Pune during the academic year 2024-2025.

| Mr. Deepak Jaiswal Industry Mentor | Dr. Kalyani Kadam Guide | Dr. Shruti Patil Head, Department of AI&ML |
| --- | --- | --- |

## DECLARATION

I hereby declare that the project titled “Autonomous AI in Navigation and Visualization” submitted to Symbiosis Institute of Technology, Constituent of Symbiosis International (Deemed University) Pune for the award of the degree of Bachelor of Technology in Artificial Intelligence & Machine Learning is a result of original research carried out by me. It is further declared that the project report or any part thereof has not been previously submitted to any University or Institute for the award of any degree or diploma.

Name(s) of Student(s)	: 	Sanskar Jadhav

PRN:  		21070126076

Degree: 	Bachelor of Technology in AI&ML

Department: 	Artificial Intelligence & Machine Learning

Title of the project	: 	Autonomous AI in Navigation and Visualization

_________________________________

(Signatures of the Students)

Date: 13/11/2024

## ACKNOWLEDGEMENT

I want to express my sincere gratitude to everyone who supported me throughout this project. First and foremost, I would like to thank my project guide, Dr. Kalyani Kadam, for her valuable guidance, encouragement, and feedback. She has been a constant source of inspiration and motivation for me.

I would also like to thank the head of the department, Dr. Shruti Patil, for providing me with the necessary facilities and resources for conducting this project. I am grateful to her for providing constant support and advice.

I want to acknowledge my industry mentor, Mr. Deepak Jaiswal, Head of New Technology Exploration at Kalyani Group, for his guidance in every project stage. He has been extremely supportive throughout this journey.

I would also like to thank my family and friends for their love, care, and support. They have always been there for me in times of need and stress. They have encouraged me to pursue my passion and achieve my goals.

Lastly, I thank Symbiosis Institute of Technology Pune for allowing me to work on this project and enhance my skills and knowledge. I am proud to be a part of this prestigious institution.

## ABSTRACT

Autonomous navigation and visualization are increasingly significant topics of recent interest and remain as the focus of intensive research in allowing AI robots to navigate and identify objects in their surroundings. Deep reinforcement learning (DRL) is instrumental for autonomous exploration that works well with optimizing decision-making under uncertainty, allowing agents to explore unmapped environments in the absence of human control. For custom object detection, YOLO continues to excel in terms of accuracy and latency, with the new Segment Anything Model 2 from Meta providing the possibility for quick annotations of multiple objects recorded in a video. This project entails hyperparameter tuning and training of the TD3 algorithm for indoor navigation in a 30x30m closed environment representing an office workspace, development of a GUI for local usage of SAM 2 to maintain data privacy, and custom training of YOLOv9 for real-time object detection to be put onboard a Unitree Go2 quadruped robot for real world testing. Experimental results show that the DRL system achieves close results to the ones of a path planner in an already known environment, thereby eliminating the need for storing a map, SAM 2 can be modified to accelerate dataset creation for YOLO training on multiple classes, and with CUDA support, inference time is minimized even when using heavier YOLO models for greater accuracy. These results are part of the ongoing development of my work on embedded autonomous AI for Unmanned Ground Vehicles (UGVs).

Keywords: Autonomous navigation, Deep reinforcement learning, TD3, Object detection, YOLO, SAM 2, UGV

## TABLE OF CONTENTS

|  |  | Page |
| --- | --- | --- |
| Certificate |  | i |
| Declaration |  | ii |
| Acknowledgment |  | iii |
| Abstract |  | iv |
| Table of Contents |  | v |
| List of Tables |  | vii |
| List of Figures |  | viii |
| List of Abbreviations |  | ix |
| CHAPTER 1: INTRODUCTION |  | 1 |
| 1.1 | Introduction | 1 |
| 1.2 | Problem statement | 2 |
| 1.3 | Scope of research | 3 |
| 1.4 | Research hypothesis | 4 |
| 1.5 | Objectives | 4 |
| 1.6 | Organization of the report | 5 |
| CHAPTER 2: LITERATURE REVIEW |  | 12 |
| 2.1 | Background | 12 |
| 2.2 | Summary of literature review and research gap | 71 |
| CHAPTER 3: SOFTWARE REQUIREMENTS SPECIFICATION |  |  |
| 3.1 | Software Tool Platform/ Tools/Framework used |  |
| 3.2 | Hardware tools |  |
| 3.3 | Work Breakdown Structure |  |
| 3.4 | Functional Requirements |  |
| 3.5 | Non-Functional Requirements |  |
| 3.6 | Project Cost Estimation |  |
| CHAPTER 4: METHODOLOGY |  | 73 |
| 4.1 |  | 73 |
| 4.2 |  | 85 |
| 4.3 |  |  |
| 4.4 |  |  |
| CHAPTER 5: RESULTS AND DISCUSSION |  | 87 |
| 5.1 | Summary of results and discussion | 163 |
| CHAPTER 6: CONCLUSION AND FUTURE SCOPE |  | 164 |
| 5.1 | Conclusion | 164 |
| 5.2 | Future Scope | 1 |
| REFERENCES (in IEEE format only) |  |  |
| APPENDICES |  |  |
|  | AIC Form (Mandatory) |  |
|  | Similarity Report (Mandatory) AI Plag Report (Mandatory) |  |
|  | Certificate from Industry |  |

## LIST OF TABLES

|  |  | Page |
| --- | --- | --- |
| Table 2.1 | xxxxxx |  |
| Table 2.2 | Xxxx |  |

## LIST OF FIGURES

|  |  | Page |
| --- | --- | --- |
| Figure 2.1 | xxxxxxxx |  |
| Figure 2.2 | xxxxxxxxxxx |  |

## LIST OF ABBREVIATIONS

Chapter 1

Introduction

  - Introduction

UGV navigation involves multiple dimensions of technologies that cover the control of the vehicles themselves, the route planning, and communications with other services. Each aspect has been positively influenced by recent developments in the field of artificial intelligence and robotics. Of all these technologies, Deep Reinforcement Learning (DRL) seems to be the most effective when optimizing decision making in unpredictable and unstable environments. Within DRL, systems enable exploration of unknown territories within which all navigation is performed autonomously and learned actions are adjusted with experience. Thus, it fits perfectly for situations which do not allow traditional mapping and planning approaches.

A critical element of autonomous navigation is environmental perception, where the UGV is required to recognize, locate and react to the presence of many different objects. Object detection is usually performed using complex CNN based machine learning models; the You Only Look Once (YOLO) framework continues to be the most prominent due to its optimized trade-off between speed and accuracy. This ability, coupled with Meta's Segment Anything Model 2 (SAM 2) creates a highly effective strategy for real-time object detection, allowing quick instance segmentation of selected objects for easier annotation of datasets. These models as a whole improve the performance of the UGV in terms of accurate custom object recognition, which is especially challenging in crowded small indoor areas like offices.

The objective of this study is to develop and optimize an autonomous navigation system for the quadruped robot Unitree Go2 working within an office-like environment, specifically a 30x30m closed space. The guidance system is based on the Twin Delayed Deep Deterministic Policy Gradient (TD3) algorithm for DRL-based navigation and integrated with YOLOv9 for real-time object detection. It intends to attain levels of performance on par with traditional path-planning methods, without pre-mapped environments. This includes the development of a local GUI for SAM 2 to ensure data sovereignty and efficient operation through rapid, precise autonomous annotation without reliance on continuous manual intervention. The navigation performance, object detection performance, and overall performance of the developed system will be validated using real-world trials with the Go2, within the confines of a closed environment, with contributions to the vast field of embedded autonomous AI applicable for UGVs.

1.2 Problem Statement

While autonomous navigation and object detection technology is progressing quickly, few substantial challenges remain to be solved in the deployment of appropriate systems that can operate in real-time and reliably within confined indoor spaces. While the path-planning traditional methods that exist yield reasonable performance in controlled environments, they highly depend on pre-mapped data, which restricts its adaptability against unknown environments as those natural environments require extensive storage provision and mapping. Thus, the challenge is to navigate autonomously and dynamically with real-time efficiency within the framework of the unmapped environment, with specific attention on an indoor workspace, where accuracy and obstacle avoidance are essentially critical, and is often characterized with several cluttered static and dynamic obstacles.

Moving a step further, even though real-time detection models such as YOLO are advanced, the processes of training and annotating the dataset have proven to be highly resource-intensive and voluminous, most especially in situations where the parameters of data collections require that local processing be done due to privacy constraints. Thus, there lies a dual prospect: efficient autonomous navigation, which could work without a map; secondly, develop a local hosted data annotation application that feeds data for training high-accuracy object detection models in situ.

  - Scope of Research

This study aims to design an end-to-end solution for autonomous navigation and object detection in a chosen constrained indoor space – an office environment with 30x30m dimensions. The research will be focused on three main approaches as described below.

Navigation and Exploration via DRL: The focus of this research area will be on improving the TD3 algorithm for Indoor Workspace Map-less Navigation specifically designed for the quadruped robot Unitree Go2. The study will compare the effectiveness of navigation using a DRL approach relative to a conventional path-planning approach.

Object Detection and Annotation: Training of the YOLOv9m model with emphasis on real-time accurate detection using specially created custom datasets unique to the workspace setting. In a bid to ensure the safeguarding of data, the system employs SAM 2, which is used locally as an annotating platform for expediting the process of developing multiple object class training datasets.

Evaluation of System Performance in Real World Settings: The system is deployed on the Unitree Go2 robot using an Nvidia Jetson Orin board with the aim of understanding how well it achieves the desired level of navigation accuracy, the latency of object detection, and general indoor scenario reliability in UGV navigation.

Thus, the scope of the research implies a long-standing commitment to algorithmic modeling, systems integration, and practical application in the real world with the intent of providing a useful and workable system for autonomous navigation in ground vehicles that will also have an application in object based assisted navigation systems.

  - Research Hypothesis

The central hypothesis of the research work is that optimizing the TD3 DRL algorithm for indoor environments along with the integration of an efficient and locally implemented object detection model would result in autonomous navigation and real-time object recognition in a UGV, analogous to that done by conventional path planners for known environments. The hypothesis is based on the idea that strict tuning of hyperparameters for the TD3 algorithm would enable a UGV to navigate without an existing map, while YOLOv9, based upon local annotations provided by SAM 2, can be capable of maintaining accuracy and low latency in object detection.

1.5 Objectives

- Development and rigorous tuning of the TD3 algorithm in such a way as to make autonomous navigation possible in an office workspace-sized 30x30 m area without pre-existing maps or external localization aids.

- Developing an in-house GUI for an autonomous annotation application using a modified version of Meta's SAM 2 model to provide rapid annotations for any custom object classes that might be relevant in the target environment using videos as input.

- Train YOLOv9 for real-time object detection with annotated datasets created with SAM 2 so that office navigation can cooccur with accurate real-time object detection without producing a latency that is too high even with larger model configurations.

- Evaluate the performance of the TD3 navigation in simulated environments by considering travel time, success to collision ratio, and robustness to changing environments.

- Test the algorithms on a real Unitree Go2 quadruped robot and obtain feedback about the system's navigation accuracy, response latencies, and consistency in object detection in more realistic spaces of interest.

These objectives jointly strive to develop a solution for scalable autonomous navigation and obstacle detection, which improves the current capabilities of UGVs in dynamic cluttered indoor environments, thus providing value for both academic research and industrial applications in autonomous robotics.

Literature Review

This system represents an unassisted exploration with no human interaction with unknown environments, extraction of points of interest, and choice of the most promising one followed by the DRL-based motion policy for reaching it without local optima. The key contributions consist of a customized strategy for goal-directed exploration that devises a novel neural network based on TD3 and integrates DRL seamlessly with global navigation to enhance adaptability with uncertain environments.

All these contributions are huge leaps in autonomous exploration and navigation, showing how DRL and techniques such as Twin Delayed Deep Deterministic Policy Gradient (TD3) are used to address difficult real-world problems.

2. Literature Review

Availability of low-cost sensors and computational devices has changed the landscape of real-time Simultaneous Localization and Mapping (SLAM) in robotics [4,5,6,7,8,9,10,11,12] with capabilities for autonomous positioning and environmental perception.

But despite all of this, the ability to find correct maps of the environment mostly with the purpose of navigation still depends on human operators or given plans [13] [14] [15]. Moreover, many autonomous exploration strategies that are based on prior maps [16] [17] [18] [19] rely on such strategies to find POI for exploration by gathering information from the edges of the map, outline free spaces inside the environment. Then, path planning is conducted, and the navigation is carried out to the chosen POIs [20] [21] [22]. Deep learning methods have facilitated a paradigm shift toward ensuring reliable robot navigation using neural networks. Robots can now carry out purposeful movements through deliberate movements directly by the output of neural networks. For example, in one work [23], the environment is mapped through learned frontier area predictions, which act as guides for the robot in its exploration process. However, the navigation module is still dependent largely on a planning mechanism. Moreover, there is a proposal for exploratory navigation based on deep Q-learning [24], where robots can be taught to masterfully navigate and maneuver within unknown settings. This is generalized to work in [25], where robots learn obstacle-avoidance policies via simulations and then test them in the real world. This paper uses discrete navigation actions to steer around obstacles while no particular goal definition is provided. On the contrary, robot actions in a continuous space were developed by iteratively learning neural networks using deep deterministic policy gradients.

References [26] and [27] detail a method that combines data about the state of the environment with the objective location of the robot to form the input to the neural network. With this information merged together, decisions made are more context-aware. Yet one must point out that the information available to the network is usually localised both in time and in space. In such situations, the local optima may pose a problem to the neural networks. This essentially means that the models do not always reach global optimality decisions while navigating and exploring their world. However, although these methods are very accurate and reliable in their respective areas, they face some practical challenges regarding safety, generalization capability, local optima, and other things, thus becoming a little challenging to deploy in practice. Therefore, methods based on neural networks seem well suitable for specific tasks but may not be the best way to integrate into general and practical solutions.

Some more pioneering approaches, such as [28], successfully integrate behavior-based navigation with planning. However, they still use local planners in motion planning, and neural networks are applied mainly for dynamic obstacle avoidance in pre-mapped environments. On the contrary, references [29] and [30], concerning references, the strategy developed is a local strategy learnt with the use of a world path planning system. Nevertheless, this system has a focus on obstacle avoidance. On top of this, it is worthwhile to mention that the system already has access to the map of the environment. These approaches commonly do not include aspects related to exploration in the scope.

In [31], the authors introduced the technique based on semantic segmentation of convolutional deep learning architectures FCN-8s and SegNet. Although they have reached a maximum accuracy of 95%, their applicability to real scenarios is pretty far behind. This paper never contemplated with any other sensor other than camera. It used a huge amount of data set for training which is hard to collect and costly to train. It has no ability to learn from the error and adapt to unseen environments.  Another highly advanced research [32] in this area has reached an unprecedented navigation precision of 98% accompanied by efficiency of 10 seconds using DL-DDPG (Disturbed Learning Deep Deterministic Policy Gradient) algorithm that is an enhanced version of DDPG algorithm. However, the research purely focuses on improvement over the base DDPG algorithm. The acquired method could only drive the bot to a known destination in a known indoor space, and the method is not capable of discovering unknown environments and reaching goals that have no prior knowledge of the environment.

| Ref | Year | Authors | Methodology | Application | Key Findings |
| --- | --- | --- | --- | --- | --- |
| [6] | 2023 | Yin et al. | Mapless local path planning using DRL | Dynamic obstacle avoidance | Real-time navigation proves effective however there are issues with stability in areas with a lot of obstacles |
| [12] | 2021 | Gao et al. | A LiDAR and DRL based path planner for mobile robots | Indoor mobile robot navigation | Given high path fidelity for known environments, the limitation arises due to lack of coverage expansion into unexplored areas |
| [15] | 2022 | Wen et al. | Active SLAM with DRL | Autonomous path planning | Exploitation is combined with SLAM and DRL; however, this approach is highly demanding in terms of processing power |
| [21] | 2018 | Shi et al. | Learning based control for an end-to-end deep reinforcement navigation system | Mobile robot navigation | Simplifies training for solutions providing direct navigation given usual conditions of operation; however the system is sensitive to changes in environment |
| [27] | 2020 | Cimurs et al. | Goal oriented DRL and path planning aided obstacle avoidance | Continuous obstacle avoidance | Supports smooth actions in a continuous space with practice, but limited in that implementation without practice leads to overfitting in simulations |
| [41] | 2021 | Cui et al. | DQN based transition model embedded reinforcement learning | Multi-agent navigation with awareness of social contexts | Self-supervised pretraining participants learn to behave socially appropriately, but it is time-consuming due to the modelling predictive coding and sequence issues involved |
| [45] | 2018 | Chen et al. | Vision-based DRL for target-driven navigation | Indoor scene exploration | Such methods are successful in static setup spaces; however, it fails to extend to environments with moving objects and complicated structures |
| [50] | 2020 | Schaul et al. | Prioritized Experience Replay (PER) | Dynamic tasks learning enhancement | Improves data efficiency for training; however, possible overconvergence of training on the states which have been accessed previously |
| [59] | 2019 | Mirowski et al. | Dueling DDQN for complex navigation | Multi-room pathfinding | Improvement by value function approximation, expending computational resources on two concurrent networks |
| [61] | 2020 | Lillicrap et al. | DDPG for continuous control | Autonomous driving | Employs action control in continuous spaces; controls are difficult to tune and not robust in variable environments |
| [67] | 2021 | Brunner et al. | Deep DRL for map-based learning | Autonomous map navigation | Indoor static maps work well; however high frequency dynamic changes adaptation is limited |
| [72] | 2018 | Zhu et al. | Target-driven visual navigation using DRL | Indoor exploration | Centralized approach works very efficiently within corridors but does not transfer well to novel configurations |
| [80] | 2020 | Taschin et al. | Proximal Policy Optimization (PPO) | High-speed autonomous navigation | Works great where the world is dynamic and continuous, though it is hyperparameter sensitive for reaching stability |

- Summary of Trends

In recent years, advancements in DRL have primarily focused on goal-oriented navigation and continuous action space control. A significant trend includes integrating SLAM with DRL to improve navigation in complex environments and prioritized experience replay (PER) for enhancing learning efficiency in tasks requiring high data diversity. Computational intensity and adaptability to dynamic, real-world conditions remain challenges across these approaches.